Imaging Multiple Sclerosis with MRI

Author: Heidi Veillette, BSc, MRT(R)(MR)

Outline:

- Intro

  - Overview of multiple sclerosis disease process

  - Prevalence of MS worldwide

  - Common patient symptoms

  - McDonald criteria for diagnosing MS

    - Role of MRI in diagnosis

- MRI Protocols

  - Overview

  - Brain protocol for diagnosis

    - Optional sequences

  - Brain protocol for follow-up imaging

  - Spine protocol

    - Optional sequences

  - Spine protocol for follow-up imaging

- Visible signs of MS on MR images

  - Dawson’s fingers

  - Central vein sign

  - Iron rim sign

  - Open ring sign

- Future Considerations and Conclusion

Learning Objectives:

- Outline the process of multiple sclerosis disease development and its associated symptoms.

- Explain the recommended MS MRI brain and spine protocols, including the benefits of each sequence and the recommended use of contrast media.

- Describe the visible signs of MS on MRI images.

Imaging Multiple Sclerosis with MRI

Multiple sclerosis (MS) is a progressive, inflammatory neurodegenerative disease of the central nervous system (CNS). It is considered an immune-mediated disease whereby the body’s immune system attacks the CNS and causes demyelination. Demyelination is when the myelin, the fatty substance surrounding and insulating nerve fibers, is damaged. The nerve fibers, then unprotected, also get damaged, and develop scar tissue lesions called sclerosis.2 These lesions can occur in many areas of the CNS, giving the disease its name “multiple sclerosis”.2 The exact mechanism of why the immune system attacks its own CNS is unknown, but environmental factors, infectious disease, and genetics are thought to combine to trigger the disease.2

According to the Atlas of MS, an online open-source collection of MS data from around the world, 2.8 million people are living with MS as of 2020. This number has increased from 2.3 million in 2013, and the cause for the increase is unknown.3 Females are two to three times more likely to have MS than males, with the average age of diagnosis around 36 years.3 Multiple sclerosis is the second most common cause of neurological impairment in young adults, after trauma.

Multiple sclerosis leads to chronic, progressive, irreversible disability in the majority of patients. Although there is still no cure, several medications are available for treatment for reducing relapse rates and severity and slowing disability progression.1

Table 1: Common symptoms in patients with MS at the time of diagnosis and during relapses or flare-ups.2,5

Diagnosing MS is challenging because patients present with varying symptoms and intensities, depending on the quantity, location, and activity of lesions in the CNS. Table 1 shows the most common symptoms that may be listed on the requisition form when a patient comes to the MRI department for their initial or follow-up MS MRI exam.2,

The correct, early diagnosis of MS is critical for beginning treatment and having an optimal prognosis. The McDonald criteria is the standard set of guidelines used by physicians for diagnosing MS. Diagnosis requires clinical testing, elimination of other possible diagnoses, and the evidence of sclerotic lesions in the CNS disseminated in space and time. Dissemination in space (DIS) means that lesions are located in multiple sections of the CNS, including the white matter, grey matter, cerebellum, brainstem, spinal cord, or optic nerve. Dissemination in time (DIS) means that there is evidence that damage occurred at different points in time.2

MRI is the optimal imaging modality for showing both DIS and DIT. It is the primary modality for MS diagnosis and follow-up imaging to determine the disease pattern, progression, and response to treatment.5 The 2017 revision of the McDonald criteria highlighted the strong need for strict standardization of MRI acquisition and interpretation to avoid misdiagnosis.

MRI MS Protocols

While there is no single regulatory body that can provide worldwide standardization of MS MRI protocols, recently published recommendations from the Magnetic Resonance Imaging in Multiple Sclerosis study group, Consortium of Multiple Sclerosis Centers, and North American Imaging in Multiple Sclerosis Cooperative (MAGNIMS-CMSC-NAIMS) include equipment specifications and specific brain and spine sequences for optimal MS imaging.9

High-field magnets of 3 Tesla (3T) or higher provide a better MS lesion detection rate and possible shorter acquisition times compared to lower-field magnets, but there is no evidence that 3T MRI provides an earlier diagnosis of MS. The use of 1.5T scanners continues to be sufficient for brain lesion detection at the time of diagnosis, as long as sequences are of good quality with adequate signal-to-noise and spatial resolution. The use of scanners with field strengths lower than 1.5T is not recommended.9 For detecting spinal cord lesions, there is no evidence that scanning at higher field strengths leads to a higher detection rate, so 1.5T scanners are adequate for all spinal MS imaging.

MRI MS protocols may vary between sites depending on radiologists’ preference and equipment specifications, but one aspect of MRI brain imaging that is universally agreed upon is angling axial slices to the anterior commissure – posterior commissure (AP–PC) line off of a sagittal image. This creates standard axial slices that can be closely compared to previous and future images.

Brain Protocol for MS Diagnosis

Multiple sclerosis lesions in the brain are best visualized on fluid-sensitive sequences such as T2 and FLAIR.5 Figure 1 shows MS lesions seen on an axial FLAIR image. The 3D FLAIR sequence is considered the “core” sequence in MRI MS protocols due to its high sensitivity in detecting lesions.9 Three-dimensional acquisitions not only have improved lesion detection over 2D acquisitions, but they also allow for adjustments to the anatomic orientation when comparing to previous MRI scans. However, in centers that are unable to acquire 3D FLAIR images with high image quality, 2D sequences with ≤3mm slice thickness and no gap can provide an acceptable alternative.9

In addition to standard 3D FLAIR and axial T2 sequences, injecting gadolinium-based contrast agents during the brain protocol for initial diagnosis is recommended. Gadolinium enhancement occurs when the blood–brain barrier is damaged during active inflammation of new lesions or reactivation of previous lesions.11 This enhancement can demonstrate dissemination in time, and exclude other diagnoses. Figure 1 shows enhancing lesions on a post-contrast T1 image.12 The time delay between contrast administration and the post-contrast axial T1 sequence should be identical during follow-up scans and not shorter than 5 minutes (ideally 10 minutes).

Figure 1: MRI images from a 31-year-old male patient with known MS who presented with slow reaction times and memory loss for three days. (A) Axial FLAIR sequence shows hyperintense periventricular lesions. (B) Axial T1 post-contrast image demonstrates enhancing uniform nodular, circular lesions. (C) Sagittal T2 fat-saturated image of the cervical and thoracic spines show multiple hyperintense lesions in the spinal cord.12

A practical and cost-effective strategy to assure a delay of 5–10 min is to inject the gadolinium at the beginning of the exam and acquire the T2 and FLAIR sequences before the post-contrast T1 sequence. The T2 and FLAIR sequences are not affected by the presence of gadolinium.9

Pre-contrast T1 sequences do not provide added value for routine clinical purposes and are therefore not required.9 The recommended MRI brain protocol for the initial diagnosis of MS is shown in Table 2.9

Optional Sequences for MS Diagnosis

Proton density sequences, while commonly performed in the past, are now considered optional since high-quality FLAIR sequences have improved the detection of juxtacortical and periventricular lesions.1

Diffusion-weighted imaging (DWI) is frequently added to brain imaging protocols for MS diagnosis and monitoring, but its value is limited. DWI should be considered to exclude other diseases during the initial diagnosis scan, but it is considered unnecessary for follow-up scans in patients with known MS.9

Susceptibility-weighted images (SWI) or T2* sequences can highlight vessels in the brain and may show a “central vein sign”, which is a hypointense vein running through a hyperintense lesion. SWI can also identify paramagnetic iron rim enhancement around chronic active lesions in 50% of MS patients. This effect rarely occurs in other neurological conditions and therefore has the potential to increase the MR specificity in differentiating MS from non-MS.15 The central vein sign and iron rim enhancement are explained later in the course.

Table 2: Recommended MRI MS brain protocols for initial diagnosis and follow-up imaging.9

Brain Protocol for Follow-up MS Imaging

Patients diagnosed with MS will generally have routine follow-up imaging every 6 to 12 months, after starting a new treatment, or during a flare-up or relapse of symptoms.11 These follow-up scans should be done on the same equipment as the initial scan, using the same sequence parameters and slice angles to ensure proper measurements.1,9

Previously, the recommended protocol in these situations included the full brain protocol with an injection of gadolinium.11 The use of gadolinium has been controversial recently, with many agencies recommending a reduction in the number of gadolinium injections due to the unknown toxic effect of multiple injections throughout the course of a lifetime. In addition, some researchers have found that with 3T scanners, the use of a gadolinium-based contrast agent did not change the diagnosis of disease progression. In addition, if clinical management will not be changed depending on the presence or absence of enhancement, the administration of gadolinium is not needed.16

The consensus in the literature is that the use of gadolinium is not recommended for standard monitoring purposes, if a comparable recent (within one year) MRI scan is available.9 New and/or enlarging lesions seen on T2 and FLAIR are reliable markers of active inflammatory disease and may be superior to enhancing lesions in many clinical situations.

The quality of 3D FLAIR images on 3T is so high that some researchers even consider additional T2 sequences to be optional.9 An abbreviated protocol with 3D FLAIR reconstructed into axial and sagittal planes and in selected cases, an added contrast-enhanced T1 sequence, is sufficient for follow-up imaging.9 Although each site has different protocols for follow-up MS imaging, the recommended abbreviated follow-up MS MRI protocol from the MAGNIMS-CMSC-NAIMS group is shown in Table 2.9

Spine Protocol for MS Diagnosis

A spinal cord MRI is important for demonstrating DIS and DIT for the initial diagnosis of MS, and for excluding other diagnoses such as cord compression, vascular disease, or inflammatory disease.14 The cervical spinal cord is most affected by MS, with the thoracic and lumbar spinal cord sections affected with decreasing frequency. Many sites will image the thoracic spine down to the conus in addition to the brain and cervical spine during the initial diagnostic scan, especially if there are lower body symptoms like urinary incontinence. In this situation, the thoracic spine protocol will usually be identical to the cervical spine protocol.

The recommended cervical spine protocol must include at least two of the following three sagittal sequences: T2, proton density (PD), or short-tau inversion recovery (STIR).9 The acquisition of one sagittal sequence has limited sensitivity in depicting signal abnormalities, so a second sequence is required to confirm the presence of lesions and exclude artifacts. Axial T2 sequences can further improve diagnostic certainty by confirming and characterizing lesions seen on sagittal images and visualizing lesions at the periphery of the spinal cord that may be missed on the sagittal images, but this sequence is not considered necessary in every exam.9

Table 3: Recommended MRI MS spine protocols for initial diagnosis and follow-up imaging.9

Figure 1 shows MS lesions in the cervical and thoracic spinal cord in a sagittal T2 fat-saturated (FS) image.12 Note that the STIR sequence is recommended to replace the T2 FS sequence in spinal imaging due to better homogenous fat saturation.

Although the occurrence of enhanced lesions is less frequent in the spinal cord than in the brain, a sagittal post-contrast T1 sequence is recommended during the initial diagnostic exam if lesions are detected pre-contrast.9 The post-contrast T1 sequence should be performed immediately after the post-contrast brain MRI if both brain and spine scans are performed in the same visit.9 The recommended spine protocol for initial MS diagnosis is shown in Table 2.9

Spine Protocol for Follow-up MS Imaging

The added value of a spine MRI for follow-up purposes is debatable and should be considered on a case-by-case basis, at the radiologists’ discretion. A follow-up spine MRI may be done if the patient presents with possible new active spinal lesions, symptoms that require an alternate diagnosis, or when monitoring a patient with spinal cord MS with no or low number of brain lesions.9, The spine protocol for follow-up MS imaging is shown in Table 3.9

Visible Signs of MS on MRI Images

Multiple sclerosis lesions are hyperintense to background tissue on the T2 and FLAIR sequences. Lesions in the brain tend to have a round or oval shape measuring ≥3 mm in the long axis and have asymmetric distribution in these three areas of the brain:6

- Periventricular: Adjacent to the walls of the lateral ventricles, extending perpendicularly outward.

- Juxtacortical or Cortical: Adjacent to the cortical band or within the cortical grey matter.

- Infratentorial: Within the brainstem, pons, or cerebellum.

Lesions in the spinal cord are often oval or cigar-shaped, smaller than two vertebral body heights, and typically located in the posterior and lateral areas of the spinal cord.6

Figure 2: Sagittal FLAIR image shows lesions perpendicular to the lateral ventricle giving the classic finger-like appearance called Dawson’s finger lesions.22

Dawson’s Finger Lesions

Dawson’s finger lesions are ovoid lesions that extend outward from the lateral ventricles on sagittal FLAIR images. Visualization of Dawson finger lesions can strongly discriminate MS from other inflammatory disorders of the CNS, with very high sensitivity, ranging from 95% to 100%., Figure 2 shows an example of Dawson’s finger lesions on a sagittal FLAIR image.

Central Vein Sign

Multiple sclerosis lesion distribution in the brain occurs in a perivascular pattern, meaning lesions tend to surround a vein. Advanced susceptibility-based techniques like T2* have made it possible to visualize small veins within the brain. Combining separate T2* images with FLAIR images to create FLAIR* images allows the simultaneous visualization of MS lesions and their central veins.23, This “central vein sign” is defined as the presence of a vein as a hypointense line or dot in at least two perpendicular planes running centrally through a hyperintense lesion.23- Example FLAIR* images showing the central vein sign can be seen in Figure 3.

The central vein sign is emerging as a valuable diagnostic marker for MS, since a high proportion of lesions with this sign suggests MS rather than its mimics. It is especially useful in certain situations such as when patients present with a clinical suspicion of MS but have atypical clinical, laboratory, or imaging findings.1 However, since optimal pulse sequences are not widely available on all clinical scanners there is insufficient evidence to recommend the routine use of the central vein sign as a diagnostic marker.9

Iron Rim Sign 

As previously mentioned, an iron rim surrounding a chronic active lesion can be visualized on SWI images. This sequence increases the visibility of tissues with iron content due to their paramagnetic properties, and this effect increases with increased field strength.1 Approximately 50% of patients with MS show at least one iron-rim lesion, while that effect is absent in non-inflammatory neurological diseases.15

SWI sequences are widely available on clinical scanners, but often not performed routinely in MS protocols. More research needs to be done to determine specific SWI sequence parameters and interpretation criteria before the iron rim sign is used as a diagnostic marker for MS.

Open ring/horseshoe sign

In post-contrast T1 brain images, the outer rim of active lesions enhances with an incomplete ring shape, often called a “horseshoe sign”. The incomplete portion of the ring usually points to the grey matter. Detection of an incomplete enhancing ring can assist in differentiating demyelination from infections, infarcts, and tumors, which have completely enhanced rings.28 The open ring/horseshoe sign has high sensitivity and specificity regarding the diagnosis of MS.28

Figure 3: 3T FLAIR* images (FLAIR images combined with T2* images) demonstrating the central vessel sign in a patient with multiple sclerosis (A). The hypointense vessel is visible on all three planes in the center of the hyperintense lesion. The central vein sign is absent in a patient presenting with a migraine (B).26

Future Considerations and Conclusion

Many aspects of MS MRI imaging are still under debate or in the initial stages of research. Advances in technology have allowed for improved image quality and a reduction in exam time with the elimination of unnecessary sequences. Further research needs to be done on determining the necessity of gadolinium injections in follow-up MRI scans and on the sensitivity of using the central vein sign and iron rim sign as standard diagnostic criteria.

MRI technologists play a key role in MS diagnosis and follow-up imaging. It is important to recognize that MS symptoms and their accompanying mobility challenges can affect each patient differently. Protocols for MS imaging may vary between sites, but it is essential to understand what each sequence demonstrates. Technologists are the primary healthcare worker who can adequately answer exam-related patient questions.

References

Rovira À, Auger C. Beyond McDonald: updated perspectives on MRI diagnosis of multiple sclerosis. Expert Rev Neurother. 2021;21(8):895-911. doi:10.1080/14737175.2021.1957832

National MS Society. National Multiple Sclerosis Society. Accessed March 28, 2023. https://www.nationalmssociety.org/

Atlas of MS. Accessed March 24, 2023. https://www.atlasofms.org/map/global/epidemiology/number-of-people-with-ms

Sarbu N, Shih RY, Jones RV, Horkayne-Szakaly I, Oleaga L, Smirniotopoulos JG. White Matter Diseases with Radiologic-Pathologic Correlation. RadioGraphics. 2016;36(5):1426-1447. doi:10.1148/rg.2016160031

Gaillard F. Multiple sclerosis | Radiology Reference Article | Radiopaedia.org. Radiopaedia. doi:10.53347/rID-1700

Weidauer S, Raab P, Hattingen E. Diagnostic approach in multiple sclerosis with MRI: an update. Clin Imaging. 2021;78:276-285. doi:10.1016/j.clinimag.2021.05.025

2017 McDonald Criteria for the Diagnosis of Multiple Sclerosis. Accessed March 28, 2023. https://nms2cdn.azureedge.net/cmssite/nationalmssociety/media/msnational/professionals/diagnosing%20criteria/2018_dxcard_2018-01.png

Reich DS, Lucchinetti CF, Calabresi PA. Multiple Sclerosis. N Engl J Med. 2018;378(2):169-180. doi:10.1056/NEJMra1401483

Wattjes MP, Ciccarelli O, Reich DS, et al. 2021 MAGNIMS-CMSC-NAIMS consensus recommendations on the use of MRI in patients with multiple sclerosis. Lancet Neurol. 2021;20(8):653-670. doi:10.1016/S1474-4422(21)00095-8

Hagens MH, Burggraaff J, Kilsdonk ID, et al. Impact of 3 Tesla MRI on interobserver agreement in clinically isolated syndrome: A MAGNIMS multicentre study. Mult Scler Houndmills Basingstoke Engl. 2019;25(3):352-360. doi:10.1177/1352458517751647

Traboulsee A, Simon JH, Stone L, et al. Revised Recommendations of the Consortium of MS Centers Task Force for a Standardized MRI Protocol and Clinical Guidelines for the Diagnosis and Follow-Up of Multiple Sclerosis. AJNR Am J Neuroradiol. 2016;37(3):394-401. doi:10.3174/ajnr.A4539

Li T, Xiao H, Li S, Du X, Zhou J. Multiple sclerosis: clinical features and MRI findings in Northern China. Eur J Med Res. 2014;19(1):20. doi:10.1186/2047-783X-19-20

Hu XY, Rajendran L, Lapointe E, et al. Three-dimensional MRI sequences in MS diagnosis and research. Mult Scler Houndmills Basingstoke Engl. 2019;25(13):1700-1709. doi:10.1177/1352458519848100

Thompson AJ, Banwell BL, Barkhof F, et al. Diagnosis of multiple sclerosis: 2017 revisions of the McDonald criteria. Lancet Neurol. 2018;17(2):162-173. doi:10.1016/S1474-4422(17)30470-2

Maggi P, Sati P, Nair G, et al. Paramagnetic Rim Lesions are Specific to Multiple Sclerosis: An International Multicenter 3T MRI Study. Ann Neurol. 2020;88(5):1034-1042. doi:10.1002/ana.25877

Zarei F, Ghaedian M, Ghaedian T. The role of contrast-enhanced and non-contrast-enhanced MRI in the follow-up of multiple sclerosis. Acta Radiol Stockh Swed 1987. 2021;62(7):916-921. doi:10.1177/0284185120946714

Wang KY, Uribe TA, Lincoln CM. Comparing lesion detection of infratentorial multiple sclerosis lesions between T2-weighted spin-echo, 2D-FLAIR, and 3D-FLAIR sequences. Clin Imaging. 2018;51:229-234. doi:10.1016/j.clinimag.2018.05.017

Philpott C, Brotchie P. Comparison of MRI sequences for evaluation of multiple sclerosis of the cervical spinal cord at 3 T. Eur J Radiol. 2011;80(3):780-785. doi:10.1016/j.ejrad.2010.09.031

Zecca C, Disanto G, Sormani MP, et al. Relevance of asymptomatic spinal MRI lesions in patients with multiple sclerosis. Mult Scler Houndmills Basingstoke Engl. 2016;22(6):782-791. doi:10.1177/1352458515599246

Raz E, Loh JP, Saba L, et al. Periventricular lesions help differentiate neuromyelitis optica spectrum disorders from multiple sclerosis. Mult Scler Int. 2014;2014:986923. doi:10.1155/2014/986923

Tan IL, van Schijndel RA, Pouwels PJ, et al. MR venography of multiple sclerosis. AJNR Am J Neuroradiol. 2000;21(6):1039-1042.

Lopaisankrit T, Thammaroj J. Brain and Spinal Cord MRI Findings in Thai Multiple Sclerosis Patients. J Imaging. 2023;9(2):27. doi:10.3390/jimaging9020027

Sati P, Oh J, Constable RT, et al. The central vein sign and its clinical evaluation for the diagnosis of multiple sclerosis: a consensus statement from the North American Imaging in Multiple Sclerosis Cooperative. Nat Rev Neurol. 2016;12(12):714-722. doi:10.1038/nrneurol.2016.166

Kolb H, Al-Louzi O, Beck ES, Sati P, Absinta M, Reich DS. From pathology to MRI and back: Clinically relevant biomarkers of multiple sclerosis lesions. NeuroImage Clin. 2022;36:103194. doi:10.1016/j.nicl.2022.103194

Mistry N, Dixon J, Tallantyre E, et al. Central veins in brain lesions visualized with high-field magnetic resonance imaging: a pathologically specific diagnostic biomarker for inflammatory demyelination in the brain. JAMA Neurol. 2013;70(5):623-628. doi:10.1001/jamaneurol.2013.1405

Solomon AJ, Schindler MK, Howard DB, et al. “Central vessel sign” on 3T FLAIR* MRI for the differentiation of multiple sclerosis from migraine. Ann Clin Transl Neurol. 2016;3(2):82-87. doi:10.1002/acn3.273

Cortese R, Magnollay L, Tur C, et al. Value of the central vein sign at 3T to differentiate MS from seropositive NMOSD. Neurology. 2018;90(14):e1183-e1190. doi:10.1212/WNL.0000000000005256

Masdeu JC, Quinto C, Olivera C, Tenner M, Leslie D, Visintainer P. Open-ring imaging sign: highly specific for atypical brain demyelination. Neurology. 2000;54(7):1427-1433. doi:10.1212/wnl.54.7.1427