Pediatric Fluoroscopy

Jessyca B. Wagner, Ph.D., RT(R), CIIP

Learning Objectives:

Upon completion of this directed reading, readers will be able to:

- EXPLAIN fluoroscopy.

- IDENTIFY the medical professionals involved in pediatric fluoroscopy.

- LIST types of fluoroscopic procedures used for pediatric patients.

- COMPARE the use of fluoroscopy in adults versus children.

- RECOGNIZE the indications for pediatric fluoroscopic exams.

Outline:

- Introduction

  - History of fluoroscopy

  - Explanation of fluoroscopy

  - Comparison of fluoroscopy to other imaging modalities

- Fluoroscopic Procedures - General

  - Gastrointestinal System

  - Biliary System

  - Genitourinary System

  - Musculoskeletal/Spinal

  - Cardiovascular/Interventional

- Fluoroscopic Procedures - Pediatric

  - Gastrointestinal

  - Genitourinary

  - Interventional

- Indications and Considerations for Pediatric Fluoroscopy

  - Indications by Procedure

  - General Pediatric Considerations

  - Radiation Safety Considerations

- Conclusion

Abstract

Fluoroscopy uses ionizing radiation to create dynamic, real-time images of the internal structures of the body. This modality is used by radiologic technologists and radiologists to aid in the detection and diagnosis of disease and injury, as well as to guide medical procedures. Fluoroscopy can be used on patients of any age and status to assess and evaluate the gastrointestinal (GI), biliary, genitourinary, musculoskeletal, neurological, and cardiovascular systems. It is important for radiologic technologists and radiologists to follow the ALARA principle and adhere to time, distance, and shielding protocols to limit radiation dose to themselves and their patients. Medical imaging professionals should wear lead aprons to limit occupational exposure while utilizing proper technique, centering, positioning, and collimation for the patient. This is especially important for the pediatric patient population, as they are more radiosensitive than adult patients. Most fluoroscopy procedures that can be performed on adult patients can also be used with pediatric patients; however, if it is possible to use ultrasound or magnetic resonance imaging (MRI) to achieve the same goal, those non-ionizing modalities should be the first choice. This article will discuss the history of fluoroscopy, common procedures and indications for adult and pediatric patients, and special considerations for the pediatric population.

Pediatric Fluoroscopy

History of Fluoroscopy

Fluoroscopy is a form of medical imaging that demonstrates movement and functionality of the internal structures of the body in real time using ionizing radiation, often with the addition of contrast media.,, This modality is also used to guide medical procedures both in and out of the medical imaging department., Fluoroscopy machines can be permanently mounted in the department or mobile, as c-arms that can be taken into the operating room, patient rooms, and emergency department. The origin of fluoroscopy can be traced back to 1896, one year after Wilhelm Roentgen discovered x-rays. Thomas Edison developed the first fluoroscope, which he coined the Edison Vitascope, just four months after the discovery of x-rays.,,,,, This device consisted of only a cardboard tube coated in fluorescent metal salt held over an x-ray tube.8,10,11 After testing over 8,000 materials, Edison determined that calcium tungstate fluoresced the brightest, which is what he used in his fluoroscope.6,7 However, the image was very dim, so the viewer had to “dark-adapt” by sitting in a darkened room for a prolonged period or wear specialized viewing goggles until the invention of the intensifying screen over 50 years later. In 1950, the intensifying screen was invented and removed the need for the viewer to dark-adapt or wear special goggles, improved the visibility of details in the image, and allowed for indirect viewing of the image when coupled with mirror optics, which provided a significant reduction in radiation dose to the operator.12 Later, the mirror optics were changed to television screens.

Also in 1896, a significant advancement was made by British physicist John Hall-Edwards, when he introduced the use of x-rays in surgery and later developed a specialized piece of equipment for use in surgery.9 In fact, in 1901, Edison’s assistant, Clarence Dally, attempted to use a fluoroscope to help locate a bullet in President McKinley’s abdomen after he was shot; however, doctors declared President McKinley too unstable to undergo the fluoroscopy, and he died a week later.11 Over the coming years, research into the field of x-rays and fluoroscopy continued and unfortunately caused many injuries and deaths among those who studied it, especially among Edison’s team.6,7,11

Edison’s pursuit of discovery using radiation halted in 1900, four years after developing the fluoroscope, when he started losing his eyesight.11 Around the same time, his assistant, Clarence Dally, developed skin lesions on his face and hand, lost his hair, eyebrows, and eyelashes, and noticed heavy wrinkles in his face, along with intense pain and swelling in his left hand.11 He had always used his left hand in the fluoroscope as his imaging test subject. When this happened, instead of stopping his pursuit of further knowledge, Dally simply switched hands.11 Dally assumed that with time away from the tubes and rest, he would heal normally. However, by 1902, the pain in Dally’s hands had intensified, and they appeared scalded. Despite many skin grafts from his legs, the lesions on his hands remained. Doctors finally determined that he had developed skin carcinoma on his left hand and Dally agreed to have the arm amputated.11 Less than one year later, doctors had to remove four of the fingers from Dally’s right hand. In 1903, Dally had to have his right arm amputated, and by 1904, at 39, his body was so ravaged with metastatic skin cancer that he died after eight years of experimenting with radiation.7,11

The loss of Dally’s life, while tragic, was not in vain, as it led to awareness of the dangers of working with radiation.7,11 However, many still debated the origin of many of the injuries and deaths in those working with x-rays, so it was many years before proper regulations were put in place.6 In the 1930s, many shoe stores were equipped with a Shoe Fitting Fluoroscope, which allowed clerks and customers to examine the effect of a shoe on the bones of the feet. After more information came to light regarding the effects of unnecessary radiation doses, these devices were banned from use in the United States in the 1960s but remained in use in parts of Britain and Canada for another decade.6

Explanation and Uses of Fluoroscopy

Fluoroscopy works much like traditional diagnostic radiography in that an x-ray beam passes through the patient, strikes an image detector, and produces an image. However, there are a few additional steps with fluoroscopy that allow the image to be dynamic instead of static. Fluoroscopy machines can be configured in two ways: one with an image intensifier and one with a digital flat panel detector. Both methods offer real-time, dynamic images of the patient’s body.10,12 In both configurations, the x-ray tube is mounted under the x-ray table, sending the radiation up through the patient. This is the opposite of traditional diagnostic radiography.10,12 The fluoroscopy imaging chain in units with an image intensifier starts with the x-ray tube and collimator, which sends the beam of radiation up through the radiolucent table into the patient. Here, the x-ray photons interact with the patient and are absorbed, transmitted, or scattered. The photons that pass through the patient strike a grid attached to the image intensifier. The image intensifier sends the beam through an optical coupling device and video camera, where it is digitized and sent to a TV monitor for viewing by the radiologist during the examination.10,12 In units with a flat panel detector, the process begins the same until the beam passes through the patient. The photons strike a grid attached to the flat panel detector, and the detector converts the analog image to a digital format, sending the information to a computer where it can be visualized on a monitor by the radiologist.10,12 Using a flat panel detector for fluoroscopy is more efficient and results in a higher quality image that can be produced with less radiation, resulting in a lower dose for the patient and the medical imaging professionals performing the exam.9,10,12

Fluoroscopy has a variety of applications and uses in the medical field to evaluate many systems in the human body. It can be used alone as its own examination, or with other diagnostic and therapeutic procedures.9,10 Most fluoroscopy exams are completed by a radiologist or radiologist assistant and assisted by a radiologic technologist. Occasionally, a radiology nurse may also be included depending on the exam.1,2 All personnel involved in fluoroscopy procedures are expected to have a certain level of training and qualifications to perform all exams safely. The physician performing the exam must have an understanding of anatomy, physiology, and pathophysiology, as well as the ability to perform clinical and imaging evaluation of patients to ensure the proper procedure is being performed. They must also meet all educational and competency requirements outlined by the American College of Radiology (ACR) to perform or supervise fluoroscopy procedures.5 The physician is assisted by the radiologist-led team which may consist of a non-physician radiology provider (NPRP) such as a registered radiologist assistant (RRA), radiologic technologist, and other ancillary personnel as needed. The NPRP, radiologic technologist, and other ancillary personnel must also meet educational and competency requirements outlined by the ACR to ensure all examinations performed are of the highest quality and exhibit exemplary patient care.5

Common uses of fluoroscopy include, but are not limited to: gastrointestinal and biliary – barium studies of the GI tract from esophagus to colon, placement and assessment of various GI tubes and catheters, examination of the biliary system, retrieval of biliary stones, and placement of catheters in the bile duct in interventional procedures;1,2,8,9,10, genitourinary – examination of the female reproductive organs, examination, catheterization, and stenting of the urinary system from kidneys to bladder, retrieval and/or lithotripsy of urinary stones;3,10,13 musculoskeletal - evaluation of bones and joints, arthrography and myelography to aid in the injection of contrast media into joint spaces or vertebral spaces to better visualize injuries with cross-sectional modalities such as magnetic resonance imaging (MRI) or computed tomography (CT), lumbar punctures, and assisting in orthopedic surgeries such as fracture repairs, spinal fusions, and joint replacements;1,2,8,9,10 and cardiovascular and interventional radiology – evaluation of the heart and vessels with angiography, arteriography, or venography, intervention of cardiac problems with angioplasty, cardiac stenting, embolization, and IVC filters, placement and assessment of ports, central lines, PICC lines, and other intravenous catheters, image-guided needle biopsies, drainage and aspiration of abscesses and fluid collections, feeding tube placement, and cancer treatment.1,2,9,10,

Comparison of Fluoroscopy with Other Imaging Modalities

While fluoroscopy use has been declining in recent years in favor of ultrasound or cross-sectional imaging modalities such as MRI and CT, its unique ability to offer the radiologist a real-time view inside the body cannot be dismissed. Traditional diagnostic radiography only offers static images of the body and modalities like MRI, CT, and nuclear medicine do not allow repositioning of the patient during the examination. However, with fluoroscopy, the physician can see what is happening inside the patient in real-time, like a movie, with dynamic images and they can reposition the patient during the exam to get a better look when an abnormality is observed.15 Fluoroscopy also uses less radiation than CT. However, fluoroscopy requires radiologists and technologists be proficiently trained in a hands-on environment, and with reimbursements for fluoroscopic procedures declining, the medical imaging professionals are often not given the opportunity to receive the training that is needed, leading to frustration and subpar images.15

General Fluoroscopic Procedures

As previously mentioned, general fluoroscopy consists primarily of gastrointestinal/biliary, genitourinary/reproductive, musculoskeletal, and cardiovascular and interventional procedures. While the reasons behind the procedures were mentioned, the procedures were not, which is the purpose of this section. This is not an exhaustive list of all procedures offered in fluoroscopy, but these are the most commonly performed exams in each area. In the tables that follow, the most common procedures with a brief description will be listed: Table 1 - Gastrointestinal (GI), Table 2 - Biliary, Table 3 - Genitourinary, Table 4 – Musculoskeletal/Spine, and Table 5 - Cardiovascular & Interventional.

Table 1 - Gastrointestinal (GI)

| Fluoroscopic Exam | Description |
| --- | --- |
| Barium Swallow (Esophagram) Single or Double Contrast | A contrast-enhanced radiographic study commonly used to assess structural characteristics of the entire esophagus |
| Modified Barium Swallow Performed with a speech pathologist | A contrast-enhanced radiographic study commonly used to evaluate a patient’s ability to swallow safely and effectively using items with different thicknesses and textures |
| Upper GI Series Single or Double Contrast | A contrast-enhanced radiographic study commonly used to assess structural characteristics of the esophagus, stomach and small intestine |
| Small Bowel Follow Through (SBFT) Can be performed after UGI or as a stand-alone exam | A contrast-enhanced radiographic study commonly used to assess structural characteristics of the small intestine |
| Barium Enema Single or Double Contrast | A contrast-enhanced radiographic study commonly used to assess structural characteristics of the large intestine (colon) |

Table 2 - Biliary

| Fluoroscopic Exam | Description |
| --- | --- |
| T-Tube Cholangiogram | An intraoperative or post-operative radiographic study used to assess biliary patency and evaluate possible biliary obstruction, leak, or stone using water-soluble contrast |
| Endoscopic Retrograde Cholangiopancreatography (ERCP) | A procedure that combines upper gastrointestinal (GI) endoscopy and contrast-enhanced fluoroscopy to treat problems of the bile and pancreatic ducts |

Table 3 - Genitourinary

| Fluoroscopic Exam | Description |
| --- | --- |
| Cystogram May be performed in radiology or in the OR | A contrast-enhanced radiographic study commonly used to assess structural characteristics of the bladder |
| Voiding Cystourethrogram (VCUG) | A contrast-enhanced radiographic study commonly used to assess structural characteristics of the bladder and evaluate possible vesicoureteral reflux. The exam is similar to a cystogram, except the patient then voids their bladder under fluoroscopy23 |
| Retrograde Urethrogram | A contrast-enhanced radiographic procedure, performed most commonly in male patients, to diagnose urethral pathology, such as trauma to the urethra or urethral stricture |
| Intravenous Pyelogram/Intravenous Urogram (IVP/IVU) | A contrast-enhanced radiographic study commonly used to assess structural characteristics of the kidneys, ureters, and bladder |
| Hysterosalpingogram (HSG) | A contrast-enhanced radiographic study, performed in female patients, to evaluate the shape of the uterus, determine if fallopian tubes are open, and investigate miscarriages because of problems in the uterus |

Table 4 – Musculoskeletal/Spine

| Fluoroscopic Exam | Description |
| --- | --- |
| Arthrogram Can be performed on any joint Combined with either MRI or CT | Fluoroscopy-guided injection of contrast media into a joint space to enhance the visibility of injury to tendons, ligaments, and cartilage; typically followed by MRI or CT, |
| Joint Aspirations Can be performed for any joint | Fluoroscopy-guided removal of fluid from a joint space, especially with suspected infection; can be performed with an arthrogram27 |
| Joint Injections Can be performed in any joint, spine, or SI joints | Fluoroscopy-guided injection of corticosteroid with or without local anesthetic into joint space, spine facets, or SI joints for pain relief |
| Lumbar Puncture | Fluoroscopy-guided removal of a small amount of cerebrospinal fluid (CSF) or injection of medication or other substance into the lumbar region of the spinal canal |
| Myelogram Cervical, Thoracic, or Lumbar | Fluoroscopy-guided injection of contrast media into the thecal sac of the spinal canal to examine the spinal cord and assess spinal and disc problems; typically followed by CT when MRI is contraindicated28, |

Table 5 - Cardiovascular & Interventional

| Fluoroscopic Exam | Description |
| --- | --- |
| Angiography | X-ray of the arteries and veins to find blockage or narrowing of the vessels, as well as other problems28 |
| Angioplasty | Minimally invasive treatment in which a catheter is used to open a blocked blood vessel to improve the blood flow |
| Cancer Treatment | Targeted injection of cancer treatment medication (such as chemotherapy) directly into the tumor14 |
| Cardiac Catheterization | Procedure in which a thin, flexible tube (catheter) is guided through a blood vessel to the heart to diagnose or treat certain heart conditions, such as clogged arteries or irregular heartbeats |
| Central Venous Access | Insertion of a tube beneath the skin and into the blood vessels so patients may receive medication or nutrients directly into the blood stream32 |
| Embolization | Procedure in which medical materials or devices are placed into a blood vessel to temporarily or permanently stop bleeding32 |
| Image Guided Biopsy | Insertion of a small needle into almost any part of the body, guided by imaging techniques, to take a tissue biopsy14 |
| IVC Filter Placement | Insertion of a preventative tool inside the inferior vena cava to trap any blood clots that may break free from the veins in the legs and potentially reach the heart or the lungs32 |
| Percutaneous Drainage | Insertion of a catheter through a small nick in the skin to the site of an infection or abscess to collect and drain the fluid32 |
| Stent Placement | Placement of a woven tube covered by a metal mesh at the site of weakness to reinforce the arterial wall32 |
| Thrombolysis | Treats vascular blockages and improves blood flow by dissolving abnormal blood clots32 |
| Tumor Ablation Microwave or Radiofrequency | Microwave ablation uses electromagnetic waves to destroy a tumor32 Radiofrequency ablation (RFA) uses radio waves to create heat and damage tissue to decrease pain by damaging pain-sensing nerves or to treat cancer by damaging tumor cells32 |
| Vertebroplasty/Kyphoplasty | Fluoroscopy-guided treatment of fractured vertebrae by injecting a cement mixture into the fractured bone through a hollow needle with (kyphoplasty) or without (vertebroplasty) the use of a balloon32, |

Pediatric Fluoroscopic Procedures

The most common pediatric fluoroscopic procedures include the evaluation of swallowing and the gastrointestinal tract using a barium swallow (esophagram), upper GI with or without SBFT, and single or double contrast barium enema; evaluation of the urinary tract with an intravenous pyelogram/intravenous urogram (IVP/IVU), retrograde urogram, and voiding cystourethrogram (VCUG); and guidance of interventional procedures.10  These procedures are typically performed much the same way; however, pediatric patients often require additional measures, such as immobilization, changes in preparation, changes in contrast dosage, changes in technical factors, and bringing in additional personnel to complete the examination quickly and efficiently.

Indications and Considerations for Pediatric Fluoroscopy

Indications by Procedure

Indications for pediatric fluoroscopy exams are sometimes the same as the indications for adult patients; however, there are also those unique to pediatric patients. Common indications for pediatric barium swallow studies include but are not limited to dysphagia, odynophagia, noncardiac chest pain, recurrent pneumonia or chronic tracheobronchial inflammation, possible esophageal stricture or esophageal foreign body or unexplained pneumomediastinum., Common indications for pediatric upper gastrointestinal (UGI) studies with or without small bowel follow through (SBFT) include but are not limited to vomiting, abdominal pain, weight loss or failure to thrive, congenital syndromes or anomalies associated with intestinal malrotation, chronic respiratory disease, including cough, preoperative evaluation prior to gastrostomy tube placement, postoperative evaluation such as to exclude leak or obstruction, and small bowel obstruction.35,36 Common indications for pediatric barium enema studies include but are not limited to intussusception reduction, neonatal distal bowel obstruction, evaluation of cecal position in neonate with equivocal upper GI and potential malrotation, evaluation for stricture after necrotizing enterocolitis, presurgical planning and possible Hirschsprung disease in infants or older children.36

While less common in pediatric patients, an intravenous pyelogram/intravenous urogram (IVP/IVU) may be performed to evaluate ureter course and integrity.36 Retrograde urethrograms, while also less common in pediatric patients, can evaluate urethral injury or stricture.36 The most common genitourinary study performed in pediatric patients is the voiding cystourethrogram (VCUG). Common indications for this exam include but are not limited to prenatal hydronephrosis and urinary tract infection, pelvis fracture with possible bladder injury, evaluation of neurogenic bladder, ambiguous genitalia, and fistula evaluation in boys with high imperforate anus.36 Fluoroscopy can also be used in pediatric patients to check g-tube, ports, and central lines, or guide interventional procedures such as those already listed above.36 Most interventional procedures require little to no change for pediatric patients other than accommodating their smaller size. Indications for pediatric interventional procedures are numerous and can vary from patient to patient. The sheer number of procedures and indications goes beyond the scope of this paper.

General Pediatric Considerations

In 2016, in the United States, the National Commission on Radiation Protection and Measurements (NCRP) released a report stating that 16% of radiation dose to children comes from medical imaging, with 3% coming from fluoroscopy.38 This is an increase from 9% total medical radiation dose in 2009.38

Pediatric patients are not just small adults. Patients in this population have unique characteristics and needs that should be addressed by those caring for them. For several reasons, medical imaging should not be deemed “one-size-fits-all” and healthcare professionals working in the radiology department must be aware of the unique elements of pediatric imaging.,39 Table 6 - Pediatric Patient Considerations highlights some elements unique to pediatric patients that technologists, radiology nurses, radiologists, and other medical imaging professionals should examine.38,40

Table 6 - Pediatric Patient Considerations

Decisions are made by caregivers and medical professionals

Different clinical manifestations and imaging appearances for similar disorders in adults

Some patients cannot talk requiring engagement with caregivers

Child-friendly environment, including team members comfortable with and capable of pediatric care

Many exams depend on sedation and anesthesia or other immobilization strategies

Wide variation of patient size, anatomy, and physiology

Potentially inadequate configurations of ionizing radiation equipment for children because most pediatric imaging is not performed in pediatric centers in the United States

Challenges with dose estimation, radiation sensitivity, and risk estimation

Proper training in pediatric care is imperative for all healthcare professionals, especially those professionals working in a non-pediatric facility.38,39,40 Technologists and other personnel must remember pediatric patients often cannot communicate what is wrong or may not understand what is happening or why they are in the hospital, which can be scary. This is particularly true in the medical imaging department because of the large machines and unfamiliar environment. Not only do the healthcare professionals in the medical imaging department need to be trained on ways to interact with pediatric patients and earn their trust and cooperation to ensure quality images are obtained during the exam, but radiologists must be trained to correctly interpret the images that are taken.40

Often, medical imaging exams are long and require the patient to remain motionless for the duration of the exam. This can be difficult for many patients, especially those in the pediatric population. For these patients, it may be necessary to immobilize or restrain them. Options for immobilizing pediatric patients include swaddling, distraction, verbal techniques, physical holding by the parent or technologist, and devices such as the Pigg-O-Stat, Velcro straps, and sand bags.41 Sedation may also be necessary for some procedures, such as MRI.40,41 It is imperative for technologists and other healthcare professionals to obtain consent from the patient, if possible, before utilizing immobilization techniques. In this instance, even if consent is given by the parent or caregiver, failure to obtain consent from the patient changes immobilization to restraint, which can increase anxiety and cause post-traumatic stress disorder (PTSD) for future imaging procedures.41

Radiation Safety Considerations

Radiation safety should be a primary concern for technologists and radiologists regardless of patient age; however, it is especially important for pediatric patients because they are more radiosensitive.37,38,39,40 Children are two to five, some even cite ten, times more sensitive to radiation exposure than adults, which is because of the large number of rapidly dividing cells and the longer life expectancy of children, increasing the likelihood of long-term effects.38,,,, Radiation doses can vary depending on the modality being used, method of dose measurement, age and size of the patient, examination performed, and technical factors used.38,39 The dose of fluoroscopy procedures is typically described using dose rates because the dose depends on imaging time and number of images taken.39 Ionizing radiation can damage DNA either directly or secondarily. Single-strand DNA damage can be repaired easily; however, double-strand DNA damage is difficult to repair completely.39 Biological effects range from immediate cell necrosis, which can lead to organism death, to cell damage, which causes prolonged effects like birth defects and cancer, the most likely to result from medical imaging exposure.39 Radiation exposure can cause two types of biological effects: deterministic and stochastic. Deterministic effects occur at a threshold of dose, typically beyond that used for diagnostic imaging, and can include epilation, skin burns, and cataracts. Stochastic effects do not have a threshold dose and instead are believed to increase in risk and severity as the radiation dose increases. These effects include radiation carcinogenesis and radiation-induced genetic damage.39

Before performing a fluoroscopy exam or other medical imaging exam with the potential for elevated doses of radiation, healthcare personnel should check the patient’s medical record for related recent studies and note the date, modality, findings, and location of the images to ensure the procedure is medically necessary. Once the exam has been performed, all available radiation dose data should be recorded in the patient’s medical record.5 To limit radiation dose, the as low as reasonably achievable (ALARA) principle should always be used, as well as the use of time, distance, and shielding.35,39,40, Radiation exposure should always be limited to as little time as possible; healthcare personnel should distance themselves as far from the primary beam as possible; and appropriate technical factors, proper centering, and collimation should be used at all times. Healthcare personnel should also shield themselves to limit occupational exposure. The guidelines on patient shielding have recently changed and patient gonadal and fetal shielding are no longer recommended. Communicating with the patients and their caregivers is important, not only to aid in cooperation and trust but also to inform them of the risks versus benefits of the exam being performed.

There have been worldwide efforts to increase education about pediatric medical imaging for both consumers and healthcare professionals through organizations such as the National Cancer Institute, the Health Physics Society, the American Academy of Pediatrics, and the American College of Radiology, as well as efforts to regulate pediatric radiation dose.39 In the United States, one major organization is the Image Gently® Alliance, which is a coalition of healthcare professionals whose goal is to change practice to provide “safe, high quality pediatric imaging worldwide”.39,  Image Gently® has a Pause & Pulse campaign related to fluoroscopy that reminds providers to pause and child-size the technique, use the lowest pulse rate possible, and consider ultrasound or MRI when possible. This suggested change in modality is one primary difference between adults and children for fluoroscopy.40, While fluoroscopy is often recommended in adult patients over a modality with more radiation, such as CT, non-ionizing modalities such as ultrasound and magnetic resonance imaging (MRI) are recommended for pediatric patients to eliminate dose altogether when possible.40

Conclusion

Fluoroscopy is a useful modality in medical imaging to aid in the detection and diagnosis of numerous diseases and injuries and to guide life-saving medical procedures. This modality is versatile and can be used with patients of all ages to evaluate many systems in the body, including gastrointestinal, biliary, genitourinary, musculoskeletal, neurological (spinal), and cardiovascular. However, care must be taken when this, or any medical imaging procedure that uses ionizing radiation, is performed on a pediatric patient. Ultrasound and magnetic resonance imaging (MRI) should be the modalities of choice when possible, with pediatric patients to eliminate any radiation dose to their more radiosensitive tissues, lessening the potential side effects. However, when fluoroscopy and ionizing radiation are needed, technologists and radiologists should always practice the ALARA principle as well as remember the rules for time, distance, and shielding to protect themselves and the patient from unnecessary radiation exposure.

References

U.S. Food and Drug Administration. Fluoroscopy. n.d. Updated February 21, 2023. Accessed May 23, 2023. https://www.fda.gov/radiation-emitting-products/medical-x-ray-imaging/fluoroscopy

U.S. Centers for Disease Control and Prevention. Radiation in Healthcare: Fluoroscopy. n.d. Updated October 20, 2021. Accessed May 23, 2023. https://www.cdc.gov/nceh/radiation/fluoroscopy.html

U.S. National Library of Medicine. Fluoroscopy. Medline Plus. n.d. Updated November 12, 2021. Accessed May 23, 2023. https://medlineplus.gov/lab-tests/fluoroscopy/

Butler, PF. Fluoroscopy Protocols. Image Wisely. n.d. Accessed May 23, 2023. https://www.imagewisely.org/Imaging-Modalities/Fluoroscopy/Protocols

American College of Radiology. ACR–AAPM Technical Standard for Management of the Use of Radiation in Fluoroscopic Procedures. 2018. Accessed May 23, 2023. https://www.acr.org/-/media/ACR/Files/Practice-Parameters/MgmtFluoroProc.pdf

University of Washington Medicine. Featured History: Fluoroscopy. Department of Radiology. n.d. Accessed May 23, 2023. https://rad.washington.edu/blog/featured-history-fluoroscopy/

University of Kansas Department of Physics. Physics and the Detection of Medical X-Rays: Fluoroscope. Physik im Kontext (PIKO). n.d. Accessed May 23, 2023. https://web.phys.ksu.edu/mmmm/piko/fluor.htm#:~:text=The%20fluoroscope%20was%20invented%20in,hazard%20related%20to%20the%20fluoroscope

Tharaga, Thushya, Vishnuga. Fluoroscopy: History. Idaho State University. n.d. Accessed May 23, 2023. https://biologyisu.wordpress.com/history-2/

Kingu M. Fluoroscopy: A History of Advancements and Applications. LinkedIn. April 14, 2023. Accessed May 23, 2023. https://www.linkedin.com/pulse/fluoroscopy-history-advancements-applications-mkoma-kingu-/

Gearing L. Pediatric Fluoroscopy: Clinical and Practical Care and Management. eRadImaging. June 15, 2022. Accessed May 23, 2023. https://www.eradimaging.com/course/985

King G. Clarence Dally – The Man Who Gave Thomas Edison X-Ray Vision. Smithsonian Magazine. March 14, 2012. Accessed May 23, 2023. https://www.smithsonianmag.com/history/clarence-dally-the-man-who-gave-thomas-edison-x-ray-vision-123713565/

Johnston JN, Fauber TL. Fluoroscopic imaging. In Essentials of Radiographic Physics and Imaging. Third Edition. Elsevier; 2020:216-236.

Houston JD, Davis M. Fundamentals of Fluoroscopy. W. B. Saunders Company; 2001. Accessed May 23, 2023. http://www.osumcradiology.org/resources/Rotation-Books/Fundamentals-of-Fluoroscopy.pdf

Johns Hopkins Medicine. Interventional Radiology. Health: Treatments, Tests and Therapies. n.d. Accessed May 23, 2023. www.hopkinsmedicine.org/health/treatment-tests-and-therapies/interventional-radiology

Shalom NE, Gong GX, Auster M. Fluoroscopy: An essential diagnostic modality in the age of high-resolution cross-sectional imaging. World J Radiol. 2020;12(10):213-230. https://doi.org/10.4329%2Fwjr.v12.i10.213

Chen A, Tafti D, Tuma F. Barium swallow. In: StatPearls [Internet]. StatPearls Publishing; 2022:1-16. Accessed May 23, 2023. https://www.ncbi.nlm.nih.gov/books/NBK493176/

RadiologyInfo. Video Fluoroscopic Swallowing Exam (VSFE)/Esophagram. RadiologyInfo.org for Patients. n.d. Accessed May 24, 2023. https://www.radiologyinfo.org/en/info/modbariumswallow

Radiology Info. Upper GI X-Ray. RadiologyInfo.org for Patients. n.d. Accessed May 24, 2023. https://www.radiologyinfo.org/en/info/uppergi

Radiology Info. Small Bowel Follow-Through. RadiologyInfo.org for Patients. n.d. Accessed May 24, 2023. https://www.radiologyinfo.org/en/info/small-bowel-follow-thru

Radiology Info. Lower GI X-Ray. RadiologyInfo.org for Patients. n.d. Accessed May 24, 2023. https://www.radiologyinfo.org/en/info/lowergi

UT Southwestern. T-Tube Cholangiogram. UT Southwestern Department of Radiology. n.d. Accessed May 25, 2023. https://www.utsouthwestern.edu/education/medical-school/departments/radiology/protocols/assets/T-Tube%20Cholangiogram.pdf

U.S. Department of Health and Human Services. Endoscopic Retrograde Cholangiopancreatography (ERCP). National Institute of Diabetes and Digestive and Kidney Diseases. Updated June 2016. Accessed May 25, 2023. https://www.niddk.nih.gov/health-information/diagnostic-tests/endoscopic-retrograde-cholangiopancreatography

Johns Hopkins Medicine. Cystography. Health: Treatments, Tests and Therapies. n.d. Accessed May 25, 2023. https://www.hopkinsmedicine.org/health/treatment-tests-and-therapies/cystography

Burks FN, Santucci RA. Urethrogram. Medscape. Updated September 24, 2018. Accessed May 25, 2023. https://emedicine.medscape.com/article/1893948-overview

Radiology Info. Intravenous Pyelogram (IVP). RadiologyInfo.org for Patients. n.d. Accessed May 25, 2023. https://www.radiologyinfo.org/en/info/ivp

Radiology Info. Hysterosalpingography. RadiologyInfo.org for Patients. n.d. Accessed May 25, 2023. https://www.radiologyinfo.org/en/info/hysterosalp

Murphy A. Arthrogram. Radiopaedia. Updated March 23, 2023. Accessed May 25, 2023. https://radiopaedia.org/articles/arthrogram-1?lang=us

Inside Radiology. For Health Professionals: Tests. n.d. Accessed May 23, 2023. https://www.insideradiology.com.au/for-health-professionals/tests/

Radiology Info. Facet Joint Block. RadiologyInfo.org for Patients. n.d. Accessed May 25, 2023. https://www.radiologyinfo.org/en/info/facet-joint-block

Radiology Info. Lumbar Puncture. RadiologyInfo.org for Patients. n.d. Accessed May 25, 2023. https://www.radiologyinfo.org/en/info/spinaltap

Radiology Info. Myelography. RadiologyInfo.org for Patients. n.d. Accessed May 25, 2023. https://www.radiologyinfo.org/en/info/myelography

Society of Interventional Radiology. Glossary of Interventional Radiology Treatments. Patient Center: Conditions and Treatments. n.d. Accessed May 25, 2023. https://www.sirweb.org/patient-center/conditions-and-treatments/glossary-of-ir-treatments/

Mayo Clinic Staff. Cardiac Catheterization. Mayo Clinic. Updated October 15, 2021. Accessed May 26, 2023. https://www.mayoclinic.org/tests-procedures/cardiac-catheterization/about/pac-20384695

Radiology Info. Vertebroplasty & Kyphoplasty. RadiologyInfo.org for Patients. n.d. Accessed May 26, 2023. https://www.radiologyinfo.org/en/info/vertebro

American College of Radiology. ACR-SPR Practice Parameter for the Performance of Contrast Esophagrams and Upper Gastrointestinal Examinations in Infants and Children. Revised 2020. Accessed May 26, 2023. https://www.acr.org/-/media/ACR/Files/Practice-Parameters/UpperGI-Infants.pdf

University of North Carolina at Chapel Hill. Pediatric Fluoro Handbook. UNC Radiology Resident Education. Updated January 1, 2021. Accessed May 26, 2023. https://rads.web.unc.edu/wp-content/uploads/sites/12234/2018/07/Peds-Fluoro-Manual-for-Residents-2020-v2.pdf

Image Gently®. Enhancing Radiation Protection During Pediatric Fluoroscopy. 2013. Accessed May 26, 2023. https://www.imagegently.org/Portals/6/Procedures/Module%201%20transcript.pdf

Frush DP, Sorantin E. Radiation use in diagnostic imaging in children: approaching the value of the pediatric radiology community. Pediatr Radiol. 2021;51(4):532-543. https://doi.org/10.1007/s00247-020-04924-6

Frush DP, Applegate KE. Radiation risk from medical imaging in children. In Medina LS, Applegate KE, Blackmore CC, eds. Evidence-Based Imaging in Pediatrics: Optimizing Imaging in Pediatric Patient Care. Springer;2010:25-39. DOI: 10.1007/978-1-4419-0922-0_3

Thukral BB. Problems and preferences in pediatric imaging. Indian J Radiol Imaging. 2015;25(4):359-364. https://doi.org/10.4103%2F0971-3026.169466

Murphy A. Pediatric immobilization. Radiopaedia. Updated March 23, 2023. Accessed May 31, 2023. https://radiopaedia.org/articles/paediatric-immobilisation?lang=us

Ploussi A, Brountzos E, Rammos S, Apostolopoulou S, Efstathopoulos E. Radiation exposure in pediatric interventional procedures. Cardiovasc Intervent Radiol. 2021;44(6):857-865. https://doi.org/10.1007/s00270-020-02752-7

Johnson MM. Radiation protection education in fluoroscopy. Radiol Technol. 2015;86(5):511-528. http://www.radiologictechnology.org/content/86/5/511.long

American Association of Physicists in Medicine. AAPM Position Statement on the Use of Patient Gonadal and Fetal Shielding. 2019. PP 32-A. Accessed May 26, 2023. https://www.aapm.org/org/policies/details.asp?id=468

Image Gently®. The Alliance. n.d. Accessed May 26, 2023. https://www.imagegently.org/About-Us/The-Alliance

Image Gently®. Pause and Pulse: Image Gently in Fluoroscopy. n.d. Accessed May 26, 2023. https://www.imagegently.org/Procedures/Fluoroscopy