Dual-Energy CT Multiple-Choice Questions

- Which x-ray photon interaction occurs more in atoms with higher atomic numbers?

  - Compton scattering

  - Photoelectric absorption

  - Hounsfield unit

  - CT number

- What measurement is used when discussing the range of energies in an x-ray spectrum?

  - keV

  - kVp

  - K-shell

  - Hounsfield Unit

- Which type of dual-energy CT scanner uses two x-ray tubes?

  - Dual-source

  - Single-source with rapid kV switching

  - Detector-based

  - Sequential scanning

- Which type of dual-energy CT scanner can provide true conventional CT images?

  - Dual-source

  - Single-source with rapid kV switching

  - Detector-based

  - Sequential scanning

- Which set of virtual monoenergetic images has the highest contrast-to-noise ratio?

  - Low-energy

  - High-energy

  - Blended

  - Virtual non-contrast

- Which set of post-processed images are combined with iodine-enhanced images to create an iodine map?

  - Virtual non-contrast

  - Blended

  - Low-energy virtual monoenergetic

  - High-energy virtual monoenergetic

- What is the purpose of viewing an iodine perfusion map in a pulmonary embolism DECT chest exam?

  - Visualizing perfusion defects in the lungs

  - Highlighting enhanced vessels

  - Visualizing the lungs with no enhancement information

  - Reducing motion artifacts

- On a post-EVT therapy head DECT scan, an area of interest appears hyperintense to the background brain tissue on the blended images. On the VNC set of images, that area is isodense with the surrounding tissue. What material is MOST likely in this area of interest?

  - Calcium

  - Fat

  - Normal brain tissue

  - Iodine

- Which post-processing application is best for reducing metallic beam hardening artifacts?

  - Virtual non-contrast

  - Virtual non-calcium

  - Low-energy virtual monoenergetic

  - High-energy virtual monoenergetic

- How does a photon-counting CT detector produce images with no electronic noise?

  - Improved post-processing algorithms

  - Directly converting x-ray photons into electrical signals

  - Increased counting of photon measurements

  - Reduction in the required contrast media dose

- Which material-specific post-processing application can best show the iodine content of a mass to demonstrate tumor blood volume and vascularity?

  - Iodine maps

  - Low-energy images

  - High-energy images

  - Virtual non-contrast images

- Which anatomic material has the highest amount of x-ray attenuation and highest CT number?

  - Calcium

  - Water

  - Fat

  - Air in lungs