Dual-Energy CT Multiple-Choice Questions

- Which x-ray photon interaction occurs more in atoms with high atomic numbers?

  - Compton scattering

  - Photoelectric absorption

  - Hounsfield unit (HU)

  - Computed tomography (CT) number

- What measurement is used when discussing the range of energies in an x-ray spectrum?

  - Kiloelectron volt

  - Kilovolt (peak) (kVp)

  - K-shell

  - HU

- Which type of dual-energy CT (DECT) scanner uses 2 x-ray tubes?

  - Dual-source

  - Single-source with rapid kVp switching

  - Detector-based

  - Sequential scanning

- Which type of DECT scanner can provide true conventional CT images?

  - Dual-source

  - Single-source with rapid kVp switching

  - Detector-based

  - Sequential scanning

- Which set of virtual monoenergetic images provides improved (or the highest) contrast-to-noise ratio?

  - Low-energy

  - High-energy

  - Blended

  - Virtual non-contrast (VNC)

- Which set of post-processed images are combined with iodine-enhanced images to create an iodine map?

  - VNC

  - Blended

  - Low-energy virtual monoenergetic

  - High-energy virtual monoenergetic

- What is the purpose of viewing an iodine perfusion map in a pulmonary embolism DECT chest examination?

  - To simultaneously visualize perfusion defects in the lungs

  - To highlight enhanced vessels

  - To visualize the lungs with no enhancement information

  - To reduce motion artifacts

- On a post-endovascular therapy head DECT scan, if an area on the VNC set of images is isodense with the surrounding tissue, what material is MOST likely in this area of interest?

  - Calcium

  - Fat

  - Normal brain tissue

  - Iodine

- Which post-processing application is BEST for reducing metallic beam hardening artifacts?

  - VNC

  - Virtual non-calcium

  - Low-energy virtual monoenergetic

  - High-energy virtual monoenergetic

- How does a photon-counting CT detector produce images with no electronic noise?

  - It uses improved post-processing algorithms.

  - It directly converts x-ray photons into electrical signals.

  - It counts more photon measurements.

  - It requires a reduced contrast media dose.

- Which material-specific post-processing application can BEST show the iodine content of a mass to demonstrate tumor blood volume and vascularity?

  - Iodine maps

  - Low-energy images

  - High-energy images

  - VNC images

- Which anatomic material has the highest amount of x-ray attenuation and highest CT number?

  - Calcium

  - Water

  - Fat

  - Air in the lungs