23007_Forensic Radiography: Post Test

- Forensic radiography is defined as the use of ionizing radiation to:

- Determine the cause of death

- Determine the time of death

- Avoid potential disfigurement of a cadaver

- Gather, analyze, and interpret evidence that may be used in courts or legal matters

- Which of the following would NOT be considered a use of forensic radiography?

  - A photographic or videotaped recording of a cadaver dissection

  - Investigation of nonfatal injuries in a case of suspected child abuse

  - Detecting a forged document

  - Identifying an art forgery

- In a PMCT scan, different types of tissues (eg, soft tissue, lung, bone, brain) can be distinguished by adjusting

  - X-ray attenuation windows

  - kVp

  - mAs

  - x-ray tube aperture

- A polygon mesh displays information the _________ of an object.

  - a Volume

  - Surface

  - Density

  - Temperature

- A study that examined the accuracy of PMCT found that it was able to correctly identify the cause of death 100% of the time for several types of common pathologies. The accuracy of PMCT was lowest for deaths involving:

  - Acute heart failure

  - Intracranial bleeding

  - Gunshot wounds

  - Pneumonia

- The use of _______________ can distinguish ferromagnetic objects from nonferromagnetic objects.

  - IV contrast

  - Attenuation windowing

  - Dual-energy CT

  - Multiplanar formatting

- Research suggests that PMCT may be more sensitive than traditional autopsy for detecting bony injuries, with the exception of the:

  - Ribs

  - b.Skull

  - Larynx

  - Spine

- Which of the following would be most appropriate for identifying potential lesions of the laryngeal skeleton in a case of suspected strangulation?

  - Dual-energy CT

  - 3D printing

  - Micro-CT

  - Air contrast

- A key limitation of water-based contrast in PMCT is associated with ___________ shortly after death.

- Liquefaction of fat

- Rigor mortis

- Increased vascular permeability

- Ectopic air deposits