23010_An Overview of Radiation Safety: Radiation Protection

Kelli Welch Haynes, Ed.D., RT(R), FASRT, FAEIRS

- Which of the following paramaters controls the quality or penetrating power of an x-ray beam?:

  - absorption characteristics of the object being radiographed

  - mAs

  - fluorescent yield

  - kVp

- Which of the following parameters controls the quantity of photons in the x-ray beam?

  - absorption characteristics of the object being radiographed

  - mAs

  - fluorescent yield

  - kVp

- Which combination of technical exposure factors is optimal to reduce patient dose?

  - Increased SID, low mAs

  - Low kVp, high mAs

  - High kVp, decreased SID

  - high kVp, low mAs

- Which dosimeter utilizes an aluminum oxide detector as the sensing material?

  - Direct Ion Storage Dosimeter

  - Pocket Ionization Chamber

  - Thermoluminescent Dosimeter

  - Optically Stimulated Dosimeter

- Which of the following is a dose reduction technique used in fluoroscopy?

  - Magnification mode

  - Automatic exposure rate control

  - Last image hold

  - Automatic brightness control

- What is the minimum source to skin distance for mobile fluoroscopy?

  - 30 cm

  - 38 cm

  - 40 cm

  - 48 cm

- What is the minimum lead equivalent for lead gloves, as stated by NCRP Report number 102?

  - .20 mm

  - .25 mm

  - .35 mm

  - .50 mm

- What is the minimum lead equivalent for lead aprons, as stated by NCRP Report number 102?

  - .20 mm

  - .25 mm

  - .35 mm

  - .50 mm

- What is the minimum fluoroscopy entrance exposure for general purpose fluoroscopy, as stated by NCRP Report number 102?

  - 8.0 cGy/min

  - 8.8 cGy/min

  - 9.0 cGy/min

  - 20.0 cGy/min

- What is the minimum fluoroscopy entrance exposure for high level control fluoroscopy, as stated by NCRP Report number 102?

  - 8.0 cGy/min

  - 8.8 cGy/min

  - 9.0 cGy/min

  - 20.0-40.0 cGy/min