Digital Radiography Image Acquisition and Evaluation
Multiple Choice Questions

- Which exposure factor controls the quality of the x-ray beam?

  - Milliampere

  - Seconds

  - Kilovolt peak

  - Source-to-image distance

- Which of the following types of x-ray beam filtration can be controlled by the technologist?

  - Inherent filter

  - Added filter

  - Compensating filter

  - Anode heel effect

- Which procedure alteration should be considered when imaging a patient with osteoporosis?

  - Reduce the kVp

  - Increase the kVp

  - Add a compensating filter

  - Change the focal spot size

- What effect is most likely to occur if the anatomy of interest is too small to fully cover the AEC chamber?

  - Underexposure

  - Overexposure

  - Increased spatial resolution

  - Grid line artifact

- What does the spatial resolution of a radiographic image describe?

  - The size of the field of view

  - The difference in densities of objects

  - The magnification of an object

  - The ability to distinguish small objects

- What is the limiting factor of spatial resolution in digital radiography?

  - Pixel size

  - Matrix size

  - Field of view size

  - Patient density

- What does a smaller pixel pitch represent?

  - Higher spatial resolution

  - Lower spatial resolution

  - Improved contrast resolution

  - Decreased contrast resolution

- What is the function of a detector element (del)?

  - Terminates the exposure when an adequate amount of radiation has been received

  - Converts x-rays into an electrical signal

  - Reduces scatter radiation

  - Alters the bit range of a system

- What does the term “OID” stand for?

  - Optical Image Density

  - Object to Image Receptor Distance

  - Object Intensity Detector

  - Operator Image Decision

- True or false: The abdomen has high subject contrast

  - True

  - False

- What does the bit depth of a system refer to?

  - The number of shades of grey each pixel can display

  - The distance between pixels

  - The range of exposure values that will produce an image

  - The detective quantum efficiency

- What is the primary factor contributing to radiographic noise on an image?

  - Quantum mottle

  - Inherent filtration

  - Collimation

  - Pixel size

- What is the most effective method of reducing scatter radiation when imaging a large body part?

  - Increasing the SID

  - Increasing the mAs

  - Adding a compensating filter

  - Using an anti-scatter grid

- Which type of grid has the lead strips angled to the x-ray beam divergence at a specific source-to-image distance?

  - Linear

  - Cross-hatched

  - Focused

  - Virtual

- Which artifact appears when an incorrect SID is used with a focused grid?

  - Grid lines

  - Overexposure

  - Grid cutoff

  - Metal artifact

- What is the purpose of the Exposure Index (EI) number?

  - To determine the patient’s radiation dose

  - To measure the ability of the IR to convert X-ray energy into an image signal

  - To measure the fill factor of the detector element

  - To demonstrate if the technique used was adequate for exposure

- When evaluating a radiographic image, what does a ‘burnt-out’ area represent?

  - Overexposure

  - Underexposure

  - Grid cut-off

  - Electronic masking

- What is the best practice for marker usage during an exam?

  - Place the lead marker on the patient’s clothing

  - Place the lead marker on the image receptor or tabletop

  - Use an electronic marker instead of a lead marker

  - Marker usage is optional

Answer key:

- c

- c

- a

- a

- d

- a

- a

- b

- b

- b

- a

- a

- d

- c

- c

- d

- a

- b